#### Сети:

ISP IP: 172.16.4.1/28 и 172.16.5.1/28
HQ-RTR: 172.16.4.4 BR-RTR: 172.16.5.5

VLAN100: 192.168.1.1/26 VLAN200: 192.168.2.1/28
VLAN999: 192.168.99.1/29
HQ-SRV: 192.168.1.10 HQ-CLI: 192.168.2.10 
BR-Net: 192.168.3.1/27 BR-SRV: 192.168.3.10

#### Alt-SW (для РЧ):

apt-get update && apt-get install -y openvswitch

#### ISP:

apt-get update && apt-get install nftables -y

cat <<'EOT' > /etc/nftables/nftables.nft

#!/usr/sbin/nft -f

flush ruleset

table ip nat {

chain postrouting {

type nat hook postrouting priority srcnat

oifname "ens19" masquerade

}

}

EOT

hostnamectl hostname ISP

systemctl enable --now nftables

echo net.ipv4.ip_forward=1 >> /etc/sysctl.conf

mkdir -p /etc/net/ifaces/ens2{0,1}

echo 'TYPE=eth' | tee /etc/net/ifaces/ens2{0,1}/options

echo '172.16.4.1/28' > /etc/net/ifaces/ens20/ipv4address

echo '172.16.5.1/28' > /etc/net/ifaces/ens21/ipv4address

#### HQ-RTR (EcoRouterOS):

username net_admin

password P@$$word

role admin

exit

ip domain-name au-team.irpo

ip name-server 192.168.1.10

ntp timezone utc+3

hostname HQ-RTR.au-team.irpo

ip pool dhcp 1

range 192.168.2.2-192.168.2.9

exit

или

ip pool fordhcp 192.168.2.2-192.168.2.9

dhcp-server 1

pool dhcp 64

mask 255.255.255.240

gateway 192.168.2.1

dns 192.168.1.10

domain-name au-team.irpo

static ip 192.168.2.10

client-id mac 1111.1111.1111

exit

exit

port te0

service-instance te1/isp-hq

encapsulation untagged

exit

port te1

service-instance te1/srv-net

encapsulation dot1q 100

rewrite pop 1

service-instance te1/cli-net

encapsulation dot1q 200

rewrite pop 1

service-instance te1/management

encapsulation dot1q 999

rewrite pop 1

exit

interface isp-hq

ip nat outside

ip address 172.16.4.4/28

connect port te0 service-instance te0/isp-hq

exit

interface srv-net

ip nat inside

ip address 192.168.1.1/26

connect port te1 service-instance te1/srv-net

exit

interface cli-net

ip nat inside

ip address 192.168.2.1/28

connect port te1 service-instance te1/cli-net

exit

interface management

ip address 192.168.99.1/29

connect port te1 service-instance te1/management

exit

ip nat pool nat 192.168.1.1-192.168.2.254,192.168.99.0-192.168.99.254

ip nat source dynamic inside-to-outside pool nat overload interface isp-hq

ip route 0.0.0.0/0 172.16.4.1 description default

interface tunnel.1

ip add 192.168.5.1/30

ip tunnel 172.16.4.4 172.16.5.5 mode gre

ip ospf authentication

ip ospf authentication-key P@$$word

exit

router ospf 1

network 192.168.5.0/30 area 0.0.0.0

network 192.168.1.0/26 area 0.0.0.0

network 192.168.2.0/28 area 0.0.0.0

network 192.168.99.0/29 area 0.0.0.0

passive-interface default

no passive-interface tunnel.1

exit

#### BR-RTR (EcoRouterOS):

username net_admin

password P@$$word

role admin

exit

ip domain-name au-team.irpo

ip name-server 192.168.1.10

ntp timezone utc+3

hostname BR-RTR.au-team.irpo

port te0

service-instance te0/isp-br

encapsulation untagged

exit

port te1

service-instance te1/br-net

encapsulation untagged

exit

interface isp-br

ip nat outside

ip address 172.16.5.5/28

connect port te0 service-instance te0/isp-br

exit

interface br-net

ip nat inside

ip address 192.168.3.1/27

connect port te1 service-instance te1/br-net

exit

ip nat pool nat 192.168.3.1-192.168.3.254

ip nat source dynamic inside-to-outside pool nat overload interface isp-br

ip route 0.0.0.0/0 172.16.5.1 description default

interface tunnel.1

ip add 192.168.5.2/30

ip tunnel 172.16.5.5 172.16.4.4 mode gre

ip ospf authentication

ip ospf authentication-key P@$$word

exit

router ospf 1

network 192.168.5.0/30 area 0.0.0.0

network 192.168.3.0/27 area 0.0.0.0

passive-interface default

no passive-interface tunnel.1

exit

#### HQ-RTR (Альт Сервер)

hostnamectl hostname HQ-RTR.au-team.irpo

timedatectl set-timezone Europe/Moscow

useradd -s /bin/bash -u 1010 net_admin

echo "net_admin:P@ssw0rd" | chpasswd

gpasswd -a net_admin wheel

echo 'net_admin ALL = (root) NOPASSWD: ALL' > /etc/sudoers.d/module2_net_admin

mkdir -p /etc/net/ifaces/{ens{19,20},vlan{100,200,999},gre1}

echo 'TYPE=eth' | tee /etc/net/ifaces/ens{19,20}/options

echo $'100\n200\n999' | xargs -i bash -c 'echo -e "TYPE=vlan\nHOST=ens20\nVID={}" > /etc/net/ifaces/vlan{}/options'

cat <<'EOT' > /etc/net/ifaces/gre1/options

TYPE=iptun

TUNTYPE=gre

TUNLOCAL=172.16.4.4

TUNREMOTE=172.16.5.5

TUNTTL=64

TUNOPTIONS='ttl 64'

EOT

echo '172.16.4.4/28' > /etc/net/ifaces/ens19/ipv4address

echo '192.168.1.1/26' > /etc/net/ifaces/vlan100/ipv4address

echo '192.168.2.1/28' > /etc/net/ifaces/vlan200/ipv4address

echo '192.168.99.1/29' > /etc/net/ifaces/vlan999/ipv4address

echo '192.168.5.1/30' > /etc/net/ifaces/gre1/ipv4address

echo 'default via 172.16.4.1' > /etc/net/ifaces/ens19/ipv4route

echo $'search au-team.irpo\nnameserver 192.168.1.10' > /etc/net/ifaces/vlan100/resolv.conf

echo nameserver 77.88.8.88 > /etc/net/ifaces/ens19/resolv.conf

sed -i '/^\( *##*\)* *\(net.ipv4.ip_forward\) *= *.*/{h;s//\2 = 1/};${x;/^$/{s//\nnet.ipv4.ip_forward = 1/;H};x}' /etc/net/sysctl.conf

service network restart

rm /etc/net/ifaces/ens19/resolv.conf

apt-get update && apt-get install -y nftables frr dnsmasq

service network restart

sed -i s/ospfd=no/ospfd=yes/g /etc/frr/daemons

cat <<'EOT' > /etc/frr/frr.conf

interface gre

no ip ospf passive

exit

!

interface gre1

ip ospf area 0.0.0.0

ip ospf authentication

ip ospf authentication-key P@$$word

no ip ospf passive

exit

!

interface vlan100

ip ospf area 0.0.0.0

exit

!

interface vlan200

ip ospf area 0.0.0.0

exit

!

interface vlan999

ip ospf area 0.0.0.0

exit

!

router ospf

passive-interface default

exit

EOT

cat <<'EOT' > /etc/nftables/nftables.nft

#!/usr/sbin/nft -f

flush ruleset

table ip nat {

chain postrouting {

type nat hook postrouting priority srcnat

oifname "ens19" masquerade

}

}

EOT

sed -i '/^\( *##*\)* *\(AUTO_LOCAL_RESOLVER\) *= *.*/{h;s//\2=no/};${x;/^$/{s//\nAUTO_LOCAL_RESOLVER=no/;H};x}' /etc/sysconfig/dnsmasq

cat <<'EOT' > /etc/dnsmasq.conf

port=0

interface=vlan200

listen-address=192.168.2.1

dhcp-authoritative

dhcp-range=interface:vlan200,192.168.2.2,192.168.2.14,255.255.255.240,6h

dhcp-host=id:01:11:11:11:11:11:11,192.168.2.10

dhcp-option=6,192.168.1.10

leasefile-ro

EOT

systemctl enable --now nftables frr dnsmasq

#### BR-RTR (Альт Сервер)

hostnamectl hostname BR-RTR.au-team.irpo

timedatectl set-timezone Europe/Moscow

useradd -s /bin/bash -u 1010 net_admin

echo "net_admin:P@ssw0rd" | chpasswd

gpasswd -a net_admin wheel

echo 'net_admin ALL = (root) NOPASSWD: ALL' > /etc/sudoers.d/module2_net_admin

mkdir -p /etc/net/ifaces/{ens{19,20},gre1}

echo 'TYPE=eth' | tee /etc/net/ifaces/ens{19,20}/options

cat <<'EOT' > /etc/net/ifaces/gre1/options

TYPE=iptun

TUNTYPE=gre

TUNLOCAL=172.16.5.5

TUNREMOTE=172.16.4.4

TUNTTL=64

TUNOPTIONS='ttl 64'

EOT

echo '172.16.5.5/28' > /etc/net/ifaces/ens19/ipv4address

echo '192.168.3.1/27' > /etc/net/ifaces/ens20/ipv4address

echo '192.168.5.2/30' > /etc/net/ifaces/gre1/ipv4address

echo 'default via 172.16.5.1' > /etc/net/ifaces/ens19/ipv4route

echo $'search au-team.irpo\nnameserver 192.168.1.10' > /etc/net/ifaces/gre1/resolv.conf

echo nameserver 77.88.8.88 > /etc/net/ifaces/ens19/resolv.conf

sed -i '/^\( *##*\)* *\(net.ipv4.ip_forward\) *= *.*/{h;s//\2 = 1/};${x;/^$/{s//\nnet.ipv4.ip_forward = 1/;H};x}' /etc/net/sysctl.conf

service network restart

rm -f /etc/net/ifaces/ens19/resolv.conf

apt-get update && apt-get install -y nftables frr

service network restart

sed -i s/ospfd=no/ospfd=yes/g /etc/frr/daemons

cat <<'EOT' > /etc/frr/frr.conf

interface gre

no ip ospf passive

exit

!

interface gre1

ip ospf area 0.0.0.0

ip ospf authentication

ip ospf authentication-key P@$$word

no ip ospf passive

exit

!

interface ens20

ip ospf area 0.0.0.0

exit

!

router ospf

passive-interface default

exit

EOT

cat <<'EOT' > /etc/nftables/nftables.nft

#!/usr/sbin/nft -f

flush ruleset

table ip nat {

chain postrouting {

type nat hook postrouting priority srcnat

oifname "ens19" masquerade

}

}

EOT

systemctl enable --now nftables frr

#### HQ-SRV:

hostnamectl hostname HQ-SRV.au-team.irpo

timedatectl set-timezone Europe/Moscow

echo 'TYPE=eth' > /etc/net/ifaces/ens19/options

echo '192.168.1.10/26' > /etc/net/ifaces/ens19/ipv4address

echo 'default via 192.168.1.1' > /etc/net/ifaces/ens19/ipv4route

apt-get update && apt-get install dnsmasq -y

cat 'OPTIONS=""' > /etc/sysconfig/dnsmasq

service network restart

cat <<'EOT' > /etc/dnsmasq.conf

no-resolv

no-poll

no-hosts

server=77.88.8.8

server=195.208.4.1

server=195.208.5.1

server=8.8.8.8

cache-size=1000

all-servers

no-negcache

host-record=hq-rtr.au-team.irpo,192.168.1.1

host-record=hq-srv.au-team.irpo,192.168.1.10

host-record=hq-cli.au-team.irpo,192.168.2.10

address=/br-rtr.au-team.irpo/192.168.3.1

address=/br-srv.au-team.irpo/192.168.3.10

cname=moodle.au-team.irpo,hq-rtr.au-team.irpo

cname=wiki.au-team.irpo,hq-rtr.au-team.irpo

EOT

systemctl enable --now dnsmasq

useradd -s /bin/bash -u 1010 sshuser

echo "sshuser:P@ssw0rd" | chpasswd

gpasswd -a sshuser wheel

echo 'sshuser ALL = (root) NOPASSWD: ALL' >> /etc/sudoers

#### BR-SRV:

hostnamectl hostname BR-SRV.au-team.irpo

timedatectl set-timezone Europe/Moscow

echo 'TYPE=eth' > /etc/net/ifaces/ens19/options

echo '192.168.3.10/26' > /etc/net/ifaces/ens19/ipv4address

echo 'default via 192.168.3.1' > /etc/net/ifaces/ens19/ipv4route

service network restart

useradd -s /bin/bash -u 1010 sshuser

echo "sshuser:P@ssw0rd" | chpasswd

gpasswd -a sshuser wheel

echo 'sshuser ALL = (root) NOPASSWD: ALL' >> /etc/sudoers

#### HQ-CLI:

hostnamectl hostname HQ-CLI.au-team.irpo

timedatectl set-timezone Europe/Moscow

nmcli connection modify 'Wired Connection 1' ipv4.dhcp-client-id 01:11:11:11:11:11:11